CASE STUDY 04

# Demonstration of Network Communication Models Using a Network Simulation Tool

Sample application: Small Office Network

# 1. Aim

To demonstrate how the OSI and TCP/IP models describe communication in a small simulated network.

# 2. Objectives

Identify the layers in the OSI and TCP/IP models.

Create a simple network in a simulator.

Observe packet flow and encapsulation.

Relate common protocols to the layers they use.

# 3. Selected Application and Overview

A small office network contains two PCs connected through a switch and a router. A simulator such as Cisco Packet Tracer or an equivalent educational tool can be used to observe connectivity and packet flow.

# 4. Requirements and Scope

| Devices | Two end devices, one switch and one router in the simulator. |
| --- | --- |
| Addressing | Assign unique IP addresses and suitable subnet masks to each network. |
| Protocols | Use Ethernet, IP, ICMP and optionally TCP/HTTP for demonstrations. |
| Evidence | Save the topology, addressing table and screenshots of successful tests. |

# 5. Method / Approach

Build a topology and inspect packets in simulation mode

# 6. Analysis and Discussion

| Area | Analysis |
| --- | --- |
| OSI model | Seven conceptual layers: Physical, Data Link, Network, Transport, Session, Presentation and Application. |
| TCP/IP model | Commonly described using Link, Internet, Transport and Application layers. |
| Encapsulation | Each layer adds control information as data moves down the stack. |
| Troubleshooting | Layer-based checks help distinguish cabling, addressing, transport and application issues. |

# 7. Procedure / Implementation Plan

1. Create topology: Place PCs and network devices and connect them with appropriate links.

2. Configure: Assign IP addresses and configure router interfaces if separate networks are used.

3. Test: Use ping between devices and confirm the expected reachability.

4. Inspect: Use simulation or packet inspection to follow ICMP or HTTP traffic.

5. Record: Map observed protocols to the relevant OSI and TCP/IP layers.

# 8. Sample Test Cases

| Test case | Input / action | Expected result |
| --- | --- | --- |
| Link status | Inspect device links | Required links show an active state. |
| IP configuration | Check each device's address and subnet | Addresses are unique and consistent with the topology. |
| Ping test | Ping a reachable host | Replies are received when routing and addressing are correct. |
| Layer observation | Inspect a packet event | Packet details show relevant addressing and protocol information. |

# 9. Result

The simulation illustrates that application data is carried through multiple protocol layers and that each layer has a distinct responsibility.

# 10. Conclusion

The OSI model is a useful teaching and troubleshooting framework, while TCP/IP describes the protocol suite used by real networks. Simulation makes packet flow easier to observe.

Prepared as an educational sample case study. Adapt platform-specific steps to the current interface and account plan.